Application Note 19

Tube Formation Assays in µSlide Angiogenesis

Related   topics:   Application   Note   27   “Data    Analysis   of   Tube

Formation Assays”.

Contents

1. General Information ............................................................................................  1

2. Material...............................................................................................................   2

3. Work Flow Overview ..........................................................................................  3

4. Preparation of the Gel and the Slide....................................................................  4

Gel Application ....................................................................................................  4

How to Adjust the Right Volume of Gel .............................................................. 5

Gelation ...............................................................................................................  5

5. Seeding Cells ......................................................................................................  5

5. Seeding Cells ......................................................................................................  6

6. Observation on the Microscope..........................................................................  7

Automatical Observation ..................................................................................... 7

Manual Observation ............................................................................................  7

7. Data Analysis and Interpretation ......................................................................... 7

8. Staining Protocol .................................................................................................  8

1. General Information

The  µ;Slide  Angiogenesis  is  designed  for  observing  tube  formation  on  an  inverse

TM

microscope.  It  can  be  used  with  all  common  gel  matrices,  such  as  Matrigel   ,

collagen gels, and hyaluronic acid gels. Only 10 µl of gel per well are needed.

The  platform  provided  by   µ;Slide  Angiogenesis  eliminates  the  meniscus  effect,

which is often observed in other well formats. Every cell on the flat gel surface is

visible with high;quality phase contrast or fluorescence microscopy.

µSlide Angiogenesis                                      Cross section of one well

This application note describes a sample setup with the µ;Slide Angiogenesis, which

TM

is being used for tube formation with endothelial cells (HUVEC) on Matrigel   .

Application Note 19

© ibidi GmbH, Version 1.3, Helga Wagner, June 25th, 2012

Page 1 of 8

Application Note 19

2. Material

Cells:

Medium:

HUVEC (PromoCell, C;12200, C;12203)

Endothelial Cell Growth Medium

(PromoCell, C;22010)

TM

BD   Matrigel     (Growth   Factor  Reduced,

#356231)

10  per well

4

50 µl per well

10 µl per well

Gel Matrix:

Slides:

Fluorescence stain:

µ;Slide Angiogenesis, ibiTreat (ibidi, 81506)

Calcein AM

1 Slide

1 ml (6.25 µg/ml)

(PromoKine, PK;CA707;80011)

Scale paper for checking

Other:

1 sheet

Detach reagent

Accutase (Promocell, C;41310)

8 ml per T75 flask

For easy handling, the wells are compatible with multi;channel pipettes. The plastic

is   compatible   with   various   fixing   solutions,   such   as   isopropanol,   methanol,

paraformaldehyde,  and  others.  The   optical  properties  of  the  plastic  bottom   are

comparable to those of glass coverslips.

Phase contrast picture with a 10x objective (arranged of 5x6 single pictures) which shows an

entire well with HUVECs that are forming a cell network. The image was taken 10 hours after

seeding. The diameter of the well is 4 mm.

Application Note 19

© ibidi GmbH, Version 1.3, Helga Wagner, June 25th, 2012

Page 2 of 8

Application Note 19

3. Work Flow Overview

Work flow of practical tasks in the lab. Here, Matrigel is thawed and filled into the lower

well of the µSlide Angiogenesis. After polymerization, cell suspension is applied to the

upper well, and then the network formation is observed microscopically.

Work  flow  of  image  processing.  Microscopic  pictures  are  taken  at  several  time  points

and  then  are  automatically  analyzed  (e.g.,  determination  of  tubes,  loops,  cell  covered

area, and branching points). This data is analyzed with statistical tests       to confirm the

result of the experiment.

Application Note 19

© ibidi GmbH, Version 1.3, Helga Wagner, June 25th, 2012

Page 3 of 8

Application Note 19

4. Preparation of the Gel and the Slide

Gel Application

Follow these steps:

TM

1)  The day before seeding cells, place the Matrigel     on ice in the refrigerator at

4°C. The gel can slowly thaw over night.

Note: Always use precooled pipet tips (4°C) for pipetting the gel!

2)  When starting the experiment, place

the vessel with the gel in a cool rack

in the laminar flow.

3)  Remove the µ;Slide Angiogenesis

from the sterile packing and place it

on a µ;Slide rack.

4)  Apply 10 µl of gel to each inner well.

Hold the pipet tip upright in the

middle of the well. This prevents the

gel from flowing into the upper well.

Pipetting Tips

To avoid air bubbles, make three up

and down movements (10 µl) with the

pipet while leaving the tip in the gel.

Then transfer 10 µl aliquots to the

wells.

Due to the high viscosity of Matrigel   ,

TM

it might be necessary to adjust the

pipet volume to more or less then 10

µl.

To control the right amount of gel,

observe the scale paper through the

filled wells. With an adequate volume,

there is no magnification or

minimization effect. If it is not correct,

adjust the volume with gel (see the

following page).

Application Note 19

© ibidi GmbH, Version 1.3, Helga Wagner, June 25th, 2012

Page 4 of 8

Application Note 19

How to Adjust the Right Volume of Gel

The volume of the inner well is exactly 10 µl. When the well contains the correct

volume,  no  magnification  or  demagnification  effect,  such  as  seen  in  the  picture

below, is observed. For visualizing the effect, hold the slide at a distance of a few

centimeters over a scale paper.

If  the  pipet  setting  of  10  µl  does  not  result  in  meniscus;free  filling,  try  slightly

different pipetting volumes and check with a scale paper to determine which setting

is adequate.

µSlide

Angiogenesis

filled

with  Matrigel.  Columns  1  and

2 contain less   than 10 µl. The

grid looks diminished. Column

3  is  filled  with  the   adequate

volume of 10 µl and shows no

shift. Columns 4 and 5 have an

excessive  volume.  The  grid  is

magnified.

Gelation

Follow these steps:

1)  After applying the gel, close the lid on the slide.

2)  Prepare a petri dish with water soaked paper towels for use as an extra

humidity chamber.

3)  Place the µ;Slide in the petri dish and close the lid.

4)  Place the whole assembly into the incubator for polymerization (30 min).

5)  In the meantime, prepare the cell suspension.

Preparing the humidity chamber

µSlide placed in the humidity chamber

Application Note 19

© ibidi GmbH, Version 1.3, Helga Wagner, June 25th, 2012

Page 5 of 8

Application Note 19

5. Seeding Cells

The number of cells seeded on the surface of the

gel is a crucial parameter for obtaining reliable

results. The cell type and size determine the

number of cells that are needed. For best results,

optimize the cell seeding number before starting an

experimental series. For a detailed description,

please see Application Note 27 “Data Analysis of

Tube Formation Assays”.

Follow these steps:

1)  For a final cell number of 10.000 cells per

well, adjust a cell suspension of

2 · 10  cells/ml. Then mix thoroughly.

5

2)  Take the µ;Slide from the incubator and place

it on the rack.

Filling the cell suspension with a

multichannel    pipet    into    the

upper wells.

3)  Apply 50 µl cell suspension to each upper well.

Keep the pipet tip upright and take care not to

touch the gel with the pipet tip.

For this step a multi;channel pipet might be

helpful.

4)  Again, control the correct volume with the scale paper, as shown above. If

not correct, then adjust the volume with cell;free medium.

5)  Close the slide with the lid. The slide is now ready for observation.

6)  After some minutes, all the cells will have sunk to the ground and will be

lying in one plane. Due to the geometry of the slides the cells on the margins

are placed on the plastic surface.

Sinking process of cells. After some minutes all of the cells have fallen to the ground.

The cross section of the well now shows two flat surfaces: the Matrigel    itself and

TM

the medium above. In comparison to normal well formats no meniscus disturbs the

excellent optical properties.

Comparison between the flat surfaces of a well in µSlide

Angiogenesis and a well from a 96well plate.

Application Note 19

© ibidi GmbH, Version 1.3, Helga Wagner, June 25th, 2012

Page 6 of 8

Application Note 19

6. Observation on the Microscope

There  are   two  possible  ways  to   collect  data  on   the  microscope,  manually   or

automatically. We recommend recording a time;lapse video to determine the time

dependency and the characteristics (e.g., maximum and stable phase) of the curve.

After this, single manual measurements are sufficient for investigating the effects of

substances on tube formation.

There is a detailed description on how to determine these time;lapse characteristics

in Application Note 27 “Data Analysis of Tube Formation Assays”.

Automatical Observation

Immediately  after  seeding  the  cells,  position  the  slide  on  an  inverted  microscope

equipped  with an  incubation  chamber  (e.g., the  ibidi heating  system). Choose  the

section  you  want to observe  on  your imaging  system  and  then start a time;lapse

recording. For HUVEC, we recommend a small magnification (4x or 10x) and a time

interval  of  5  minutes  in  between  the  single  images.  Use  a  software  autofocus

program  to  get  sharp  pictures  over  an  elapsed  time.  It  is  possible  that  cells  will

migrate into the gel and change the focal plane.

Manual Observation

When you know the curve of the network formation, it is sufficient to take pictures

at the most interesting time points. Incubate the slide inside of a humidity chamber

in the incubator. Take it out at distinct time points to manually take pictures on the

microscope.

Time Lapse pictures with a 10x magnification at 0, 2, and 4 hours.

7. Data Analysis and Interpretation

The pictures are analyzed based on different parameters, such as tube length, loops,

or cell;covered area. There is a detailed description about data analysis and

interpretation in Application Note 27 “Data Analysis of Tube Formation Assays”.

Application Note 19

© ibidi GmbH, Version 1.3, Helga Wagner, June 25th, 2012

Page 7 of 8

Application Note 19

8. Staining Protocol

Follow these steps:

1)  Photograph the wells before staining. This gives you a comparison of the cell

pattern before and after staining.

2)  Carefully discard the supernatant. Take care not to damage the gel or the cell

network.

3)  Add 50 µl serum;free medium with diluted calcein (12.5 µl calcein stock

1 µg/µl) at a final concentration of 6.25 µg/ml (1:160).

4)  Incubate in the dark for 30 minutes at room temperature.

5)  Wash with PBS three times. Rinse the PBS slowly over the side of the upper

well. Don’t pipette it directly onto the cells. Remove it from the other side of

the well, so that it very gently rinses the cells.

6)  Take fluorescence pictures at 485 nm/ 529 nm.

HUVEC network stained with calcein (6.25 µg/ml)

Application Note 19

© ibidi GmbH, Version 1.3, Helga Wagner, June 25th, 2012

Page 8 of 8